Kyle Dillard

SOHO Network Documentation

Brief Summary

Designed a Small Office/Home Office (SOHO	) network using Cisco Packet Tracer. The network supports two segmented VLANS for users and printers, provides inter-VLAN routing, and basic security configurations. This project demonstrates foundational networking concepts such as IP addressing, VLANS, routing, and device configuration.

IP Addressing Table for End Devices

| VLAN | Purpose | Subnet | Gateway |
| --- | --- | --- | --- |
| 10 | User PC | 192.168.10.8/24 | 192.168.10.1 |
| 10 | User PC | 192.168.10.9/24 | 192.168.10.1 |
| 10 | Printer | 192.168.10.10/24 | 192.168.10.1 |
| 20 | User PC | 192.168.20.11/24 | 192.168.20.1 |
| 20 | User PC | 192.168.20.12/24 | 192.168.20.1 |
| 20 | Printer | 192.168.20.13/24 | 192.168.20.1 |

IP Addressing Table for Router Interfaces

| Router Interface | VLAN | IP Address | Subnet |
| --- | --- | --- | --- |
| Fa0/0 | 10 | 192.168.10.1 | 255.255.255.0 |
| Fa0/1 | 20 | 192.168.20.1 | 255.255.255.0 |

Device Configuration Summary

Router2

- Default gateways configured on ports Fa0/0 and Fa0/1

- Allows for inter-VLAN connection

- Console and VTY passwords configured

- Enable secret used for privileged access

- Unused switchports administratively shut down

- Banner MOTD configured

Switch0

- Facilitates communication for VLAN 10

- Access port assigned to port Fa0/24

- Console and VTY passwords configured

- Enable secret used for privileged access

- Unused switchports administratively shut down

- Banner MOTD configured

Switch1

- Facilitates communication for VLAN 20

- Access port assigned to port Fa0/24

- Console and VTY passwords configured

- Enable secret used for privileged access

- Unused switchports administratively shut down

- Banner MOTD configured

Reflection

The completion of this project reinforced my understanding of VLAN segmentation, default-gateways and inter-VLAN routing. I gained hands-on experience troubleshooting connectivity issues related to missing gateway configurations and learned how proper network documentation improved maintainability.